About flash BIOS with ME part to support haswell refresh CPU

- Firstly, enter bios setup, setting ME firmware flash as Enabled. (Following pictures below step by step.)

- Prepare a USB Flash which can support DOS booting.

- Copy the FPT tool and BIOS file onto the USB Flash

- Connect the USB Flash to back I/O on MB. Turn on pc and press F7 continuously, select the USB Flash on boot Menu.

- Run First.bat file

- After flashing completed in step 5, then run Second.bat file.

- After flashing completed in step 6, remove the AC power for several seconds, re-connect the AC power.